# EXPERIMENTAL DESIGN, MATERIALS AND METHODS

Wind Tunnel

All the experiments in this work were performed in an open-circuit low speed subsonic wind tunnel facility available at Aerodynamics Laboratory of SASTRA Deemed University. The tunnel encompasses a rectangular closed test section featuring cross-section of 300×300×1500 mm. The test-section walls are enclosed by transparent acrylic sheets on 2 sides for better optical visualization and covered by polished wood on top and bottom. The wind tunnel is driven by a fan which is energized through 10HP motor and can attain a peak velocity around 60 m/s. The schematic illustration of experimental arrangement utilized in the present study is depicted in Fig. 2. For the purpose of measurements, a coordinate system was defined with its origin located at the airfoil leading-edge in the wind tunnel where the horizontal component (x-axis) corresponds to the free-stream flow direction, the vertical component (y-axis) denotes the vertical orientation and the lateral component (z-axis) represents the lateral direction.

Figure 2. Experimental Arrangement of the Wind Tunnel: A Schematic View.

Figure 3. representation of pressure ports in the test models

Test Airfoil:

In the present study, the test model made of PLA (Polylactic acid) material with a symmetrical airfoil profile NACA 63(4)-021 was utilized. The test models have a chord length of 100mm which is mounted horizontally within the test section supported by an aluminium holder extending through the walls of the test section. This rod acts as a pitch control mechanism secured in place using a mechanical locking arrangement. The angle of attack was adjusted using a mechanical rotation mechanism at the model mount, allowing controlled variation of the airfoil incidence relative to the freestream. Hence, it is worth noting that the reported values of angle of attack correspond to the geometric angle of attack.

| (a) |  |
| --- | --- |
| (b) | (c) |
| (d) | (e) |
| (f) | (g) |
| Figure 4. Schematic representation of the Test model: (a) suction side (b) pressure side of M1 model-top view (c) pressure side of M1 model-perspective view (d) pressure side of M2 model-top view (e) pressure side of M2 model-perspective view l (f) pressure side of M3 model-top view (g) pressure side of M3 model- perspective view |  |

The NACA 63(4)-021 airfoil features a maximum thickness of 21%. The nomenclature of NACA 63(4)-021 airfoil is as follows:

6 – indicates 6-series, optimized for laminar-flow characteristics.

3 – specifies the location of minimum pressure at approximately 30% of the chord from the leading edge.

(4) – denotes design lift coefficient (0.4).

021 – indicates a maximum thickness of 21% of the chord.

It is important to emphasize that the design lift coefficient refers to the aerodynamic condition for which the pressure distribution is optimized, and it does not inherently imply geometric camber. In other words, the design lift coefficient is the operating lift coefficient at which the airfoil is aerodynamically optimized to achieve minimum drag through favourable pressure gradients and maximum laminar flow development which is not essentially at 0o angle of attack. Hence, it need not be cambered. Also, it is worth mentioning that the CAD geometry used for fabrication was generated from the standard symmetric coordinate definition of NACA 63(4)-021, ensuring zero camber throughout the chord. The models were distributed with pressure ports on both sides of the airfoil in order to capture detailed information about the surface pressure distribution and flow characteristics. The representation of distribution of the pressure ports over the modified stepped airfoils are shown in Fig. 3. Entire set of pressure ports drilled over the surface of the airfoil have an approximate diameter of 1mm.

Step configuration:

The first model (M1) incorporates three discrete steps on the pressure side of the airfoil; each step has a width of 50mm with a span length of 300mm. The M1 test model is instrumented with 64 pressure ports in total with 24 ports on the suction side of the airfoil and 40 ports on the pressure side of the airfoil. Figure 4 illustrates the schematics of the test models utilized in this study. Models M2 and M3 featured smaller-step configurations: M2 has six steps on the pressure side of the airfoil with a step width of 26.5mm while M3 has eight steps on the pressure side of the airfoil with a step width of 20mm. Both M2 and M3 are featured with a total of 61 pressure taps where 24 pressure taps on the suction side and 37 pressure taps on the pressure side of the model. Since the model is quasi-two-dimensional with a constant spanwise cross-section, the pressure ports distributed only in the chordwise direction are considered for the aerodynamic force measurements. Additionally, spanwise pressure ports were also placed in the test model especially, near the stepped configurations aiming at understanding the underlying flow physics. However, the data pertaining to spanwise pressure ports are out of the scope of the present study. Therefore, the pressure taps situated along the chordwise directions alone are utilized in the present study. The detailed list of exact chordwise and spanwise locations has been provided in ANNEXURE I as well as a separate Excel file in the repository [13].

Turbulence Grids

Turbulence in the flow was introduced using a self-designed passive turbulence grid developed in-house. Based on the framework of previous investigations [12], several grid configurations and sizes were tested and the final version employed in this study comprises parallel arrays of cylindrical rods. To achieve multiple turbulence intensities, the self-designed passive grids were positioned at 3 distinct upstream locations. In other words, the airfoil leading edge is located at a fixed distance L=750mm from the inlet and the grid is moved to different positions, the relevant downstream spacing governing grid-generated turbulence decay is (L- ZLE) i.e., the distance between the grid and the airfoil. To ensure physically meaningful normalization for grid-generated turbulence, this spacing has now been nondimensionalized using the grid rod diameter (d)=30mm. The corresponding turbulence intensities along with their respective measurement positions were tabulated in Table 1. Following the placement of the grids at their respective locations, turbulence intensities were measured using a seven-hole probe in conjunction with simultaneous pressure scanner. Turbulence grid, measurement positions, and model-grid arrangement within the test-section are illustrated in Fig. 5 & 6.

Figure 5. A schematic representation of turbulence grid location and its measurement position in wind tunnel

Figure 6. Schematics of the model location and grid location in the test section of the wind tunnel

The probe was positioned at the model centre location within the test section to ensure that the measured turbulence level corresponds to the actual flow conditions experienced by the test model. The 7-hole probe is pneumatically connected to the MPS4264 simultaneous pressure scanner of Scanivalve Make. A total of 10,000 data samples corresponding to 700 Hz sampling frequency was measured. The instantaneous velocity components were obtained from the measured pressure signals using the manufacturer-provided calibration matrix for the probe. Assuming incompressible flow, the instantaneous velocity magnitude was determined from the dynamic pressure using Bernoulli’s relation:

Vt=2∆p(t)ρ

The mean velocity (V) was computed as:

V=1Ni=1NVti

The root-mean-square (RMS) of the velocity fluctuations was calculated as:

Vrms=1Ni=1N(Vti-V)2

The turbulence intensity was then determined as

TI=VrmsV×100%

This procedure provides a time-resolved estimate of the freestream turbulence level at the model location. The integral turbulent length scale (l), representing the characteristic size of the energy-containing eddies was estimated from the autocorrelation function:

ρτ=Rτu2

Where, Rτ=ut+u(t+τ) and τ is the time lag. The integral time scale Tu was obtained from the aera under the autocorrelation curve  ρτ, which is approximated as:

Tu=0τoρτdτ

Where ρτo=0. The integral length scale was then calculated as l=VTu. This methodology is consistent with the framework described by Dos Santos et al. [14].

Table.1 Grid position & its corresponding Turbulence Intensity (TI)

| Grid Position | (L-ZLE)/d | TI (%) | Integral length scale (l) (in mm) |
| --- | --- | --- | --- |
| Without Grid | - | 0.44 | 198.9 |
| Location 1 | 25 | 6.81 | 285.7 |
| Location 2 | 21.67 | 8.58 | 290.3 |
| Location 3 | 15.00 | 10.38 | 248.6 |
| Location 4 | 11.67 | 12.59 | 243.8 |

Simultaneous pressure scanner:

Figure 7. MPS4264 Miniature Pressure Scanner of Scanivalve make

The instantaneous surface pressure over the airfoil surface were measured through the pressure taps which is pneumatically connected to MPS4264 Scanivalve pressure scanner as shown in Fig. 7. It has a range of 10” of H20 water column with ±0.06% Full-scale, long-term accuracy as per the calibration certificate provided by the original equipment manufacturer (OEM). The time-series data obtained from the Scanivalve pressure scanner were acquired at a sampling frequency of 700Hz corresponding to 10000 data samples. The model was tested at Re=2.48 x 105across varying turbulence intensities (Table 1) and angles of attack (0o to 60o in the increments of 5o) and the corresponding time-series pressure data was obtained from each of these test conditions.

The pressure data obtained from the pressure scanner could be utilized to calculate the coefficient of pressure (CPi) at ith pressure tap using the equation:

CPi= Pi-PPt-P

The instantaneous pressure acting over the pressure taps were denoted as Pi and the distance between the measurement tap being Si. A schematic representation of the aerodynamic forces over the measurement taps is shown in Fig. 8.

Figure 8. A schematic representation of the aerodynamic forces over the measurement taps

The pressure acting over the airfoil can be resolved to obtain the lift and the drag force, meaning that the surface pressure acting over the airfoil are integrated to obtain aerodynamic force coefficients acting over the test models for various angle of attack. This method is called pressure integration technique [15]. The calculation formulas utilized are as follows:

FD= (ΔP)×Si×cosα+θπ180

FL= (ΔP)×Si×sinα+θπ180

CD= FD0.5ρv2s

CL =FL0.5ρv2s